# Determination of Absolute and Relative Humidity of Atmospheric Air

## Introduction

Humidity refers to the concentration of water vapor present in the air [1]. Atmospheric air is a mixture of dry air and water vapour, and the moisture content of air can be expressed in several ways such as relative humidity, degree of saturation, and humidity ratio [2].

The measurement of atmospheric humidity using a hygrometer or psychrometer is based on the evaporative cooling principle [3]. In a wet- and dry-bulb hygrometer, two thermometers are used: the dry-bulb thermometer and the wet-bulb thermometer. The dry bulb measures the actual air temperature, while the wet bulb has its bulb covered with a moist cloth. When air passes over the wet cloth, water evaporates and absorbs latent heat from the thermometer bulb, causing the wet-bulb temperature to decrease.

If the air is dry, evaporation occurs rapidly and the wet-bulb temperature becomes much lower than the dry-bulb temperature. When the air is highly humid, evaporation occurs slowly and the difference between the two temperatures becomes small. This temperature difference is used to determine the humidity of air using psychrometric relations.

The relative humidity (RH) is defined as the ratio of the partial pressure of water vapour present in the air to the saturation vapour pressure at the same temperature.

RH=PwPsd×100

Where Pw is the partial pressure of water vapour in air, and Psd is the saturation vapour pressure at the dry-bulb temperature.

The degree of saturation (μ) is another measure of humidity and is defined as the ratio of the actual humidity ratio of moist air to the humidity ratio of saturated air at the same temperature and pressure.

μ=PwPs×P-PsdP-Pw

Where P is the total atmospheric pressure, Ps is the saturation pressure at the given temperature, Psd is the saturation pressure at dry-bulb temperature, and Pw is the partial pressure of water vapour.

The humidity ratio (w) or specific humidity represents the mass of water vapour per unit mass of dry air and is given by

ω=0.622PwP-Pw

Where 0.622 is the ratio of the molecular weight of water vapour to that of dry air.

The Carrier equation is a psychrometric relation used to determine the partial pressure of water vapour in moist air from the measured dry-bulb and wet-bulb temperatures. It is based on the principle that evaporation from the wet bulb causes cooling, and the temperature difference between the two thermometers is related to the moisture content of air.

Pw=Psw-(P-Psw)(td-tw)Kw-tw

Where Pw is the partial pressure of water vapour in air, Psw is the saturation vapour pressure at the wet-bulb temperature, td is the dry-bulb temperature, tw is the wet-bulb temperature, and Kw is the psychrometric constant

## Objective

- To measure the dry-bulb and wet-bulb temperatures of atmospheric air using a hygrometer.

- To determine the relative humidity of atmospheric air.

- To determine the degree of saturation of moist air.

- To calculate the humidity ratio of the air.

## Operation Principle

The hygrometer operates based on the principle of evaporative cooling. It consists of two thermometers: a dry-bulb thermometer, which measures the actual air temperature, and a wet-bulb thermometer, whose bulb is wrapped with a moist cloth.

When air flows over the wet cloth, water evaporates from it. During evaporation, latent heat is absorbed from the surroundings, causing the wet-bulb temperature to decrease. The amount of temperature drop depends on the moisture content of the air.

If the air is dry, evaporation occurs rapidly and the wet-bulb temperature becomes much lower than the dry-bulb temperature. If the air is already humid, evaporation occurs slowly and the temperature difference becomes small. By measuring this temperature difference and using psychrometric relations, the relative humidity, humidity ratio, and degree of saturation of the air can be determined.

## Result

| Data |  |  |  |  | Result |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Barometric Pressure |  |  |  |  | Pw | RH |  | ω |  | μ |  |
| Obs. | Td | Tw | Psd | Psw | Eqn-4 | Eqn-1 | Chart | Eqn-3 | Chart | Eqn-2 | Chart |
| 1. | 27 | 21 | 3.567 | 2.487 | 2.096 | 58.76 | 60 | 0.0131 | 0.013 | 0.83 | 0.90 |
| 2. |  | 22 |  |  | 2.161 | 60.58 | 61 | 0.0135 | 0.014 | 0.86 | 0.95 |
| 3. |  | 21 |  |  | 2.096 | 58.76 | 60 | 0.0131 | 0.013 | 0.83 | 0.90 |

Psd=3.567 KPa (given)

Psw=2.487 bar (given)

Kw=1537.8°C (given)

## Discussion

In this experiment, the humidity of atmospheric air was determined using the dry-bulb and wet-bulb temperatures obtained from the hygrometer. The wet-bulb temperature was observed to be lower than the dry-bulb temperature due to the cooling effect produced by the evaporation of water from the wet cloth. The difference between these two temperatures indicates the moisture content of the air.

Using the measured temperatures and psychrometric relations, the partial pressure of water vapour, relative humidity, degree of saturation, and humidity ratio were calculated. If the temperature difference between the two thermometers is large, it indicates lower humidity in the air, whereas a small temperature difference indicates higher humidity.

## Conclusion

In this experiment, the dry-bulb and wet-bulb temperatures of atmospheric air were measured using a hygrometer. The difference between these temperatures was used to determine the moisture content of the air. Using the psychrometric relations and the Carrier equation, the partial pressure of water vapour, relative humidity, humidity ratio, and degree of saturation were calculated.

The results show that the temperature difference between the dry and wet bulbs is directly related to the humidity of the air. Thus, the experiment successfully demonstrated a practical method for determining the absolute and relative humidity of atmospheric air using psychrometric principles.

## Reference

[1]	“Humidity,” Wikipedia. Feb. 08, 2026. Accessed: Mar. 15, 2026. [Online]. Available: https://en.wikipedia.org/w/index.php?title=Humidity&oldid=1337332503

[2]	G. Yaciuk, “SOLAR CROP DRYING,” in Solar Energy Conversion II, Elsevier, 1981, pp. 377–396. doi: 10.1016/B978-0-08-025388-6.50049-0.

[3]	“https://byjus.com/physics/hygrometer/,” BYJUS. Accessed: Mar. 15, 2026. [Online]. Available: https://byjus.com/physics/hygrometer/